PRODUCT REQUIREMENTS SPECIFICATION // SEC-CPS-2026-V1

GHOST-SPECTRUM: Autonomous RF Intrusion Detection & Multi-Band Failover Framework

An Edge-AI Driven Cyber-Physical Radio Resilience Architecture with Hardware-in-the-Loop Air-Gap Isolation

| Document Classification: | Product Requirements Document (PRD) / Technical Specification |
| --- | --- |
| System Architecture: | Tri-Node Distributed Wireless HIL (ESP32 x2 + Arduino Uno + Agent Backend) |
| Primary Target Bands: | 2.400 - 2.4835 GHz ISM (Primary) // 433.92 MHz Sub-GHz (Survival Out-of-Band) |
| Core Implementation Domains: | Layer-2 802.11 Frame Parsing, RF Signal Processing, Agentic SecOps Orchestration |

1. Executive Summary & Project Vision

Modern edge computing, smart industrial automation, and robotic manufacturing rely heavily on 2.4 GHz wireless communication standards (IEEE 802.11b/g/n Wi-Fi, Bluetooth, Zigbee). However, these systems have a critical single point of failure: Layer-1/2 wireless spectrum denial. Malicious 802.11 de-authentication floods, beacon exhaustion, and RF jamming induce an endless connection-starvation loop in standard edge OS stacks, completely blinding critical telemetry.

Ghost-Spectrum resolves this architectural vulnerability through an autonomous, cyber-physical radio resilience framework. The project eliminates analog sensor drift and calibration bottlenecks by operating strictly on deterministic digital radio registers and 802.11 link-layer frame structures. By fusing high-speed promiscuous frame monitoring with physical carrier energy sweeping across the 2.4 GHz spectrum, Ghost-Spectrum delivers real-time situational awareness to an autonomous SecOps agent on an edge server. Upon classifying an adversarial attack, the system executes an automated protocol failover to an un-jammable 433 MHz Sub-GHz survival channel, while simultaneously tripping a mechanical relay to physically air-gap vulnerable circuits.

[CORE RESEARCH PARADIGM] Ghost-Spectrum does not simulate network security in software sandboxes; it implements a Hardware-in-the-Loop (HIL) distributed system where real radio frames and physical electromagnetic anomalies trigger deterministic agentic mitigation, audited via live telemetry streaming and tactile physical actuation.

2. Problem Statement & Threat Model

2.1 The Vulnerability of 2.4 GHz Edge Infrastructure

Layer-2 Management Frame Spoofing: Because legacy 802.11 management frames (Deauth type 0x00C0, Disassociation type 0x00A0) remain unencrypted in WPA2 networks, an attacker can inject spoofed management frames using the AP's BSSID, forcing continuous disconnection across client nodes.

Layer-1 Physical RF Jamming: Adversaries can transmit continuous wave (CW) RF carriers or broadband pseudo-random noise across the 2.4 GHz band, elevating the noise floor past the receiver's sensitivity threshold (-85 dBm) and inducing 100% packet drop.

The Client Starvation Loop: When jammed, standard edge firmware (FreeRTOS, Linux wpa_supplicant) stalls in reconnection timeouts, blocking local execution threads and sensor telemetry.

2.2 Threat Classification Matrix

| Operational State | Physical RF Signature | Layer-2 Frame Signature | System Mitigation Action |
| --- | --- | --- | --- |
| NOMINAL | Normal background noise (< -80 dBm), intermittent 2.4 GHz channel bursts | Standard beacon density (~10 fps/AP), balanced data frames, 0 deauths | Maintain primary high-speed Wi-Fi telemetry uplink. Standby OOB. |
| BENIGN CONGESTION | Local energy spikes on specific channels (Bluetooth audio, microwave leakage) | Low/zero deauth frame velocity, regular beacon timing, occasional CRC retries | Maintain Wi-Fi link. Dynamically adapt payload batching. Suppress alarms. |
| L2 SPOOFING ATTACK | Low to moderate RF carrier energy (transmitter is packet-based) | Deauth/Disassoc frame velocity > 150 fps; targeted MAC dispersion focused on Gateway | Agent confirms adversarial intent. Trigger Sub-GHz OOB failover. Air-gap relay. |
| BROADBAND JAMMING | Massive uniform energy floor elevation across all 125 NRF24 channels (RPD high) | Total absence of decodable 802.11 frames (CRC failure across all channels) | Agent identifies physical layer denial. Immediate 433 MHz failover. Isolate circuit. |

3. System Goals & Academic Research Objectives

RQ 1 (Multi-Layer Sensor Fusion): Evaluate the reduction in false-positive failovers by fusing 802.11 promiscuous packet headers with NRF24L01 channel energy detection under benign interference (Bluetooth audio, file transfers).

RQ 2 (Agentic vs. Rule-Based Triage): Compare an autonomous, context-aware reasoning agent (evaluating target MAC dispersion, frame arrival velocity, and spectral entropy) against conventional static threshold controllers (e.g., if deauth > 50 then trip).

RQ 3 (Out-of-Band Resilience): Quantify packet delivery ratio (PDR) and communication continuity before, during, and after sustained 2.4 GHz denial-of-service attacks, proving that 433 MHz Sub-GHz channels maintain mission-critical telemetry.

RQ 4 (System Latency Budget): Measure and profile the end-to-end temporal latency budget: T_total = T_detect + T_classify + T_agent + T_failover, targeting a total mitigation response under 300 milliseconds.

RQ 5 (Hardware-Enforced Air-Gap): Demonstrate the electrical and logical isolation achieved by tripping the mechanical relay, preventing compromised firmware or electrical fault propagation to critical actuators.

4. Hardware Bill of Materials & Component Allocation

| Subsystem Node | Component Name | Hardware Role | Interface Bus | Operating Voltage |
| --- | --- | --- | --- | --- |
| Node 1: Sentinel | ESP32 DevKit (Type-C) | Promiscuous 802.11 Layer-2 frame sniffer; telemetry pipeline to laptop agent | UART / USB-Serial (921600 baud) | 5V USB (Internal 3.3V) |
| Node 1: Sentinel | NRF24L01+ 2.4GHz Module | Sweeps 125 RF channels, reads Received Power Detector (RPD) register | Hardware SPI (VSPI: Pins 18, 19, 23) | 3.3V Regulated (Pin 3V3) |
| Node 2: Mission Node | ESP32 DevKit (Type-C) | Primary mission telemetry node; hosts OLED and Sub-GHz failover transmitter | Wi-Fi STA + Hardware I2C + GPIO | 5V USB (Internal 3.3V) |
| Node 2: Mission Node | 0.96" SSD1306 OLED | Local link health display (2.4GHz OK vs 433MHz Failover Active) | I2C (SCL: GPIO 22, SDA: GPIO 21) | 3.3V / 5V VCC |
| Node 2: Mission Node | FS1000A 433MHz TX | Transmits authenticated OOB failover burst packets under Wi-Fi denial | 1-Wire GPIO (Pin 17) via RadioHead | 5V VCC (for max RF power) |
| Node 3: Air-Gap Node | Arduino Uno R3 + XY-MK-5V RX + 1-Ch Relay | Isolated hardware subsystem; receives 433MHz packet and mechanically trips relay | Digital Pin 11 (RX) + Pin 7 (Relay) | 5V Regulated VCC |

5. Detailed Hardware Interconnect Schematics & Pinouts

5.1 Node 1: Spectrum Sentinel (ESP32 #1 + NRF24L01)

Connect the NRF24L01 to ESP32 #1 VSPI bus:

VCC: NRF24 VCC -> ESP32 3V3 (dedicated 3.3V rail)

GND: NRF24 GND -> ESP32 GND (common system ground)

CE: NRF24 CE -> ESP32 GPIO 4 (active HIGH for RX/TX operation)

CSN: NRF24 CSN -> ESP32 GPIO 5 (active LOW SPI select)

SCK: NRF24 SCK -> ESP32 GPIO 18 (VSPI SCK clock line)

MOSI: NRF24 MOSI -> ESP32 GPIO 23 (VSPI MOSI data line)

MISO: NRF24 MISO -> ESP32 GPIO 19 (VSPI MISO data line)

5.2 Node 2: Mission Telemetry Node (ESP32 #2 + OLED + 433MHz TX)

OLED Power: OLED VCC -> ESP32 3V3 or VIN (5V); GND -> ESP32 GND

OLED I2C: OLED SCL -> ESP32 GPIO 22; OLED SDA -> ESP32 GPIO 21

FS1000A Power: FS1000A VCC -> ESP32 VIN (5V for maximum RF output power); GND -> GND

FS1000A Data: FS1000A DATA -> ESP32 GPIO 17 (RadioHead RH_ASK modulator line)

Antenna: ANT Pad -> 17.3 cm straight wire (quarter-wavelength whip antenna)

5.3 Node 3: Air-Gap Subsystem (Arduino Uno + 433MHz RX + 5V Relay)

Receiver Power: XY-MK-5V VCC -> Arduino 5V; GND -> Arduino GND

Receiver Data: XY-MK-5V DATA -> Arduino Digital Pin 11 (RH_ASK interrupt capture)

Relay Power: Relay VCC -> Arduino 5V; GND -> Arduino GND

Relay Control: Relay IN -> Arduino Digital Pin 7 (active LOW/HIGH trigger)

Air-Gap Load: Relay COM & NC -> Wired in series with simulated operational circuit power/data line

6. Data-Link, Network & RF Protocol Specifications

6.1 802.11 Layer-2 Sniffing Frame Filter

typedef struct { uint16_t frame_control; // Protocol version, Type, Subtype, Flags uint16_t duration_id; uint8_t addr1[6]; // Destination MAC Address uint8_t addr2[6]; // Source MAC Address (Transmitter) uint8_t addr3[6]; // BSSID (Access Point Identifier) uint16_t seq_ctrl; } __attribute__((packed)) wifi_mgmt_hdr_t; // Frame Control Subtype Extraction: uint8_t type = (frame_control & 0x0C) >> 2; // Management = 0x00 uint8_t subtype = (frame_control & 0xF0) >> 4; // Deauth = 0x0C (0x00C0) // Disassoc = 0x0A (0x00A0) // Beacon = 0x08 (0x0080)

6.2 NRF24L01 125-Channel RPD Carrier Sweep

uint8_t rpd_energy_bins[125]; for (uint8_t ch = 0; ch < 125; ch++) { nrf_set_channel(ch); nrf_listen_start(); ets_delay_us(150); rpd_energy_bins[ch] = nrf_read_register(RPD_REG) & 0x01; nrf_listen_stop(); }

6.3 Telemetry Stream Protocol (Sentinel to Edge Server)

{ "seq": 4182, "timestamp_ms": 104280, "channel": 6, "deauth_count": 184, "disassoc_count": 2, "beacon_count": 12, "data_count": 8, "target_bssid": "34:85:18:A2:B4:90", "unique_source_macs": 1, "nrf_energy_ratio": 0.12, "spectral_entropy": 0.38 }

6.4 433 MHz Sub-GHz Out-of-Band (OOB) Binary Frame

+-------------------------------------------------------------------------+ ¦ BYTE 0: PREAMBLE SYNC (0xAA) ¦ ¦ BYTE 1: PROTOCOL VERSION & FLAGS (0x01 = FAILOVER COMMAND) ¦ ¦ BYTE 2: SOURCE NODE ID (0x02 = ESP32 MISSION CONTROLLER) ¦ ¦ BYTE 3: INCIDENT STATUS CODE (0x55 = AIRGAP ISOLATE & SWITCH TELEMETRY) ¦ ¦ BYTE 4-5: ROLLING TELEMETRY SEQUENCE COUNTER (uint16_t) ¦ ¦ BYTE 6-7: CRC-16-CCITT CHECKSUM (Polynomial: 0x1021) ¦ +-------------------------------------------------------------------------+

7. Edge Signal Processing & Anomaly Classification Engine

Shannon Spectral Entropy (H): Evaluates the distribution of RF carrier energy across the 125 NRF24 channels. A wideband noise jammer elevates all channels uniformly, driving entropy toward 1.0 (maximum disorder), whereas standard Wi-Fi activity produces localized peaks on channels 1, 6, and 11 (low entropy): H(X) = - SUM_{i=1 to 125} [ P(c_i) * log2(P(c_i)) ], where P(c_i) = E_i / SUM(E_j).

Management Frame Arrival Velocity: Calculated over a sliding 500 ms window: V_deauth(t) = [ N_deauth(t) - N_deauth(t - delta_t) ] / delta_t. Values exceeding 50 frames/sec indicate automated script injection rather than natural connection drops.

Target MAC Dispersion Ratio: Measures the ratio of distinct transmitter MAC addresses to total deauth frames: D_mac = Distinct_Source_MACs / Total_Deauth_Frames. A low dispersion ratio (1 or 2 MACs flooding hundreds of frames) flags an automated rogue attacker.

8. Autonomous SecOps Agent Architecture (The Brain)

[ STATE: MONITORING ] <----------------------------------------------+ ¦ ¦ ¦ (Anomaly: Velocity > 50 fps OR Entropy > 0.85) ¦ v ¦ [ STATE: TRIAGE & REASONING ] ¦ ¦ ¦ +------+------+ ¦ ¦ ¦ ¦ v v ¦ [ BENIGN ] [ ADVERSARIAL ATTACK VERIFIED ] ¦ ¦ ¦ ¦ ¦ ¦--> Step 1: Issue Failover Command via 433 MHz ¦ ¦ ¦--> Step 2: Trip Physical Air-Gap Relay ¦ ¦ ¦--> Step 3: Switch OLED Status to [FAILOVER ACTIVE] ¦ ¦ ¦--> Step 4: Dispatch Incident Post-Mortem Log ¦ ¦ v ¦ ¦ [ STATE: AIR-GAPPED OOB SURVIVAL ] ¦ ¦ ¦ ¦ ¦ ¦ (Manual / Automated Recovery Command) ¦ +-------------------->+--------------------------------------------+

Tool 1 (trigger_subghz_failover): Instructs Node 2 to spin up the FS1000A transmitter and route ongoing telemetry through 433 MHz.

Tool 2 (trip_hardware_airgap): Transmits an authenticated actuation burst to the Arduino Uno, commanding Pin 7 to de-energize the relay.

Tool 3 (emit_forensic_incident_report): Generates a cryptographically hashed JSON incident record documenting attack onset, duration, BSSIDs, velocity, and entropy.

9. Hardware-in-the-Loop (HIL) Physical Air-Gap Mechanics

Normally Closed (NC) Topology: The critical simulated circuit is wired in series through the Common (COM) and Normally Closed (NC) terminals of the relay module.

Galvanic Disconnection: Under nominal conditions, the coil is unpowered, maintaining continuous contact. Upon receiving the authenticated 433 MHz command, the Arduino asserts Pin 7 HIGH, snapping the contact open.

Physical Verification: The mechanical contact separates in ~10 ms, establishing an open-circuit resistance exceeding 100 Megaohms that cannot be bypassed by software or RF exploits.

10. Mission Control Dashboard & Visual Architecture

Panel A (2.4 GHz RF Spectrum Waterfall): Live color-graded heatmap rendering real-time signal power across all 125 channels from the NRF24L01 sweeper.

Panel B (802.11 Frame Density Gauges): Real-time dual-line chart tracking Data Frames vs. Deauth bursts per second. Flashes amber during anomalies and crimson during active attack states.

Panel C (Agent Chain-of-Thought Console): Live terminal view streaming the agent's reasoning trace in real time: '[AGENT]: Frame anomaly detected -> Entropy: 0.92 -> Ruling out Bluetooth noise -> L2 Deauth attack verified -> Tripping relay.'

Panel D (Telemetry Continuity Graph): Live visual chart showing the 2.4 GHz telemetry line collapsing to 0% while the 433 MHz line picks up monotonically with zero packet loss.

Panel E (Digital Twin & Hardware State): Interactive 2D schematic mirroring desktop hardware: turns the relay status indicator red and mirrors the physical OLED readout.

11. Safe Attack Simulation & Evaluation Choreography

Minute 0:00 - 1:00 (Nominal Baseline): ESP32 #2 streams live telemetry over Wi-Fi. OLED displays 'LINK: 2.4GHz [OK]'. Dashboard renders normal blue telemetry line.

Minute 1:00 - 2:00 (Benign Interference Test): Presenter streams audio via Bluetooth or pairs the HC-05 module. Waterfall displays 2.4 GHz RF energy spikes, but deauth velocity remains zero. Agent logs: 'High RF energy on 2402-2480 MHz. Deauth velocity == 0. Classifying as BENIGN BLUETOOTH NOISE. Alarms suppressed.'

Minute 2:00 - 3:00 (Adversarial Injection): A controlled deauth burst is sent targeting ESP32 #2's MAC address from a test script. Deauth velocity spikes to >200 fps. The dashboard flashes RED.

Minute 3:00 - 4:00 (Autonomous Snap & Actuation): Agent confirms adversarial intent. The 433 MHz failover packet is dispatched. Audible SNAP: the mechanical relay trips loudly on the desk. The OLED immediately updates to 'ALERT: AIR-GAP ENGAGED -> 433MHz FAILOVER'.

Minute 4:00 - 5:00 (Continuity Verification): Telemetry continues uninterrupted on the dashboard via the 433 MHz stream. The agent automatically exports an incident post-mortem log.

12. 48-Hour Rapid Build Timeline & Milestones

| Time Window | Target Deliverable | Verification Milestone |
| --- | --- | --- |
| Day 1 (Hours 0 - 6) | Sentinel Firmware (ESP32 #1 Promiscuous) & NRF24 SPI Sweeper | ESP32 captures deauth packets and outputs JSON frame data over serial at 921600 baud. |
| Day 1 (Hours 6 - 12) | Air-Gap Node (Arduino Uno + 433MHz RX + Relay) & Mission Node (ESP32 #2) | Arduino Uno decodes authenticated 433 MHz packet and mechanically snaps relay in <15 ms. |
| Day 2 (Hours 12 - 18) | FastAPI Agent Backend & WebSocket Stream Pipeline | Python backend ingests serial telemetry, calculates rolling entropy, and triggers failover tools. |
| Day 2 (Hours 18 - 24) | Next.js / HTML5 Mission Control UI & End-to-End System Integration | Live dashboard displays spectrum waterfall, frame gauges, agent reasoning logs, and relay snap. |

13. Risk Management, Failure Modes & Mitigations

Risk 1 (ESP32 Buffer Saturation): Under sustained deauth packet floods (>1000 fps), the ESP32 promiscuous callback ring buffer can overflow. Mitigation: Allocate a 64 KB FreeRTOS ring buffer in internal SRAM, pin the sniffer task to Core 0, and offload JSON serialization to Core 1.

Risk 2 (433 MHz Ambient Static): The super-regenerative XY-MK-5V receiver floats to high gain when idle, picking up ambient electromagnetic noise. Mitigation: Mandate RadioHead RH_ASK Manchester 4b/6b encoding with 16-bit CRC checksums. Frames lacking strict 8-byte structure are discarded by hardware interrupts.

Risk 3 (Relay Coil Back-EMF Transients): Switching high inductive loads through the relay can cause back-EMF spikes that reset the Arduino. Mitigation: Use the optocoupled input on the relay board with its onboard flyback protection diode powered from a stable 5V rail.